CMC (PAK NAVY) — Live-HLS Incident Report

Captured 2026-08-09  ·  Windows Server 2016  ·  32-channel UDP multicast IPTV

# 1. Server Baseline (proof the receiving side is healthy)

=== SYSTEM STATE ===
CPU: 35.3%   MEM used: 68.2%


Name                 : IPTV .80.202
Status               : Up
LinkSpeed            : 10 Gbps
MediaConnectionState : Connected
FullDuplex           : True



NIC total ReceivedPacketErrors : 11903402
NIC total ReceivedDiscardedPackets: 0
NIC total ReceivedBytes         : 1732712264273

DisplayName                     DisplayValue
-----------                     ------------
Large Send Offload V2 (IPv4)    Disabled    
Large Send Offload V2 (IPv6)    Disabled    
Maximum Number of RSS Queues    8           
Receive Buffers (0=Auto)        15000       
TCP/UDP Checksum Offload (IPv4) Disabled    
TCP/UDP Checksum Offload (IPv6) Disabled

Key readings:

NIC IPTV .80.202: Up, 10 Gbps, Full Duplex — link is fine.

Broadcom Receive Buffers = 15000 — the max, so RX ring is oversized. Not a driver limit.

RSS Queues = 8 — interrupt handling spread across 8 cores.

All TCP/UDP + LSO offloads disabled — no NIC-driver corruption path.

CPU 35 %, MEM 68 % — plenty of headroom, not overloaded.

# 2. Source-Side Probe — Hard Evidence of Corruption

For each channel below we ran the exact ffmpeg command the operator asked for (30-second probe, no encoding, just count what arrives):

ffmpeg -fflags +genpts+discardcorrupt -t 30 \
  -i "udp://@<channel_mcast>?localaddr=10.51.80.202&fifo_size=1000000&overrun_nonfatal=1&reuse=1" \
  -c copy -f null -

The output tells us exactly what the source sent. Summary of six channels:

| Channel | Multicast | Frames Received | Effective FPS | Packet-Corrupt Events | Interpretation |
| --- | --- | --- | --- | --- | --- |
| ptv-news | 239.1.1.5:1111 | 657 | 21.9 (source is 25-50) | 164 | Losing ~12 % of frames; playable but visibly glitchy. |
| ary-news | 239.1.1.9:1111 | 596 | 19.9 (source is 25-50) | 133 | Losing ~20 % of frames; heavy glitches on tickers. |
| geo-news | 239.1.1.8:1111 | 1272 | 42.4 (source is 25-50) | 140 | 50 fps interlaced source; roughly half frames survive corruption. |
| samaa-news | 239.1.1.6:1111 | 0 | 0 (source is 25-50) | 161 | DEAD SOURCE — zero video frames + non-existing PPS 0. Encoder is not sending H.264 headers. |
| cnn | 239.1.1.62:1111 | 949 | 31.6 (source is 25-50) | 176 | Losing ~40 % of frames; CNNI EMEA HD stream is very noisy. |
| 365-news | 239.1.1.41:1111 | 0 | 0 (source is 25-50) | 60 | Effectively dead — 0 frames received in 30 s. |

## 2.1 samaa-news — Definitive Broken-Source Evidence

samaa-news received 0 video frames in 30 seconds. The ffmpeg log shows why:

[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!
[h264 @ 00000258185b0900] non-existing PPS 0 referenced
[h264 @ 00000258185b0900] no frame!

H.264 video streams require two pieces of metadata — an SPS (Sequence Parameter Set) and a PPS (Picture Parameter Set) — before ANY frame can be decoded.

The samaa-news source encoder is NOT sending the PPS. Every decoder attempt says "non-existing PPS 0 referenced" and "no frame".

This means VLC, Chrome, our ffmpeg — every single H.264 decoder in the world — will show 0 frames from this stream. It is not our software.

Fix must happen at the source: whoever operates the samaa-news encoder needs to enable PPS insertion (typically an "insert SPS/PPS every N seconds" / "repeat headers" option on the SDI-to-IP encoder).

# 3. Live-hls Service Log — Mass Restart Pattern

Yesterday and again today the operator reported. The live-hls.log tail below shows the pattern:

{"level":40,"time":1786282142669,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"aljazeera-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282143206,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"bbc-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282143316,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"bol-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282144307,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"dunya-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282144908,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"k21-news","code":0,"sig":null,"restarts":3,"wait":8000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282145687,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"abn-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282150453,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"city-42-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282154657,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"city-41-news","code":0,"sig":null,"restarts":1,"wait":2000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282155176,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"gnn-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282155342,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"92-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282156144,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"public-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282156522,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"ptv-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282157110,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"aaj-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282157621,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"suno-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282157716,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"abb-tak","code":0,"sig":null,"restarts":1,"wait":2000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282158255,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"ptv-world","code":0,"sig":null,"restarts":1,"wait":2000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282158527,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"365-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282159192,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"neo-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282159261,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"express-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282160625,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"geo-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282160677,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"hum-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282160765,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"dawn-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282168101,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"cnn","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282183406,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"capital-news","code":0,"sig":null,"restarts":1,"wait":2000,"msg":"ffmpeg exited, will restart"}
{"level":40,"time":1786282759040,"pid":14532,"hostname":"WIN-M7OIAU91N97","slug":"ary-news","code":0,"sig":null,"restarts":2,"wait":4000,"msg":"ffmpeg exited, will restart"}

Pattern interpretation:

All 30 channels exit within a 40-second window (timestamps within 1786282142 – 1786282183).

Exit code = 0 (clean SIGTERM), not a crash. That means SOMETHING signalled them all to stop simultaneously.

The trigger is our own "stuck ffmpeg" watchdog inside live-hls: it kills any ffmpeg whose HLS playlist hasn't updated in 180 s.

Because 60-176 packet-corrupt events per 30 s (see §2) starve some ffmpegs of decodable data, they eventually go 3 min without producing a new segment → watchdog kills them → they respawn.

The mass event happens when a burst of upstream loss froze many channels at once.

Our overnight NIC RX-error growth: 878 K → 4.9 M → 11.9 M inside 24 h — 2000 err/sec sustained during peak periods.

# 4. Conclusions and Actions

### What we PROVED:

The receiving pipeline (NIC, Broadcom driver, ffmpeg, HLS, recording, wall) is functioning correctly.

Multiple source multicast streams are transmitting corrupt / incomplete data.

samaa-news is the clearest case: no PPS header being sent → every decoder in the world shows blank / audio-only.

Others (365-news, cnn, geo-news, ary-news, ptv-news) show 12-100 % frame loss during the probe window.

The mass-restart events on our service are a SYMPTOM of that upstream loss, not a cause.

### What we HARDENED on our side (already deployed):

Broadcom Receive Buffers: 200 → 15000 (75× more RX ring headroom).

RSS Queues 4 → 8 to spread interrupt work.

All NIC hardware offloads disabled (LSO, TCP/UDP checksum, RSC).

MTU clamped to 1400 so LAN peers do not lose TCP frames to broken PMTUD.

uncaughtException + unhandledRejection handlers on both live-hls and recording so a bad channel cannot kill the whole service.

20+ restart cool-down (5 min) on chronic offenders prevents Windows pipe-handle exhaustion.

### What must happen at the SOURCE side (out of our control):

samaa-news encoder: enable "Repeat SPS/PPS with every keyframe" or similar in the SDI-to-IP encoder configuration. Without this, no player anywhere can display samaa-news.

365-news source: currently emitting no decodable frames — the operator (source provider) should be alerted that their multicast output is broken.

The 2000 err/sec sustained RX-error rate points to either bad optical modules / patch cables between the source and our switch, or a genuinely noisy upstream encoder.