CMC BROADCAST MONITORING PLATFORM

Production Infrastructure Specification

Server & Video-Wall Hardware Recommendation

| Prepared for | Client Review |
| --- | --- |
| Prepared by | Analysys |
| Document version | 1.0 |
| Date issued | 05 August 2026 |

CONFIDENTIAL  ·  INTERNAL DISTRIBUTION

© 2026 Analysys  ·  Prepared for the Pakistan Navy

# 1. Executive Summary

This document specifies the recommended production hardware for the Channel Monitoring & Clipping (CMC) platform — the broadcast intelligence system that ingests thirty-two DGPR-approved television channels, records them twenty-four-by-seven for a rolling thirty-day retention window, and drives the operations-room video wall.

The specification is designed for continuous production operation with a built-in fifty per-cent capacity margin — headroom for additional channels, future AI/analytics features, and unplanned peaks in clip-render or archive-playback workload. The proposed configuration will comfortably sustain the current 32-channel line-up while leaving room to grow to 48 channels without further hardware investment.

Two categories of hardware are required: (a) a central server that performs ingest, storage, encoding, and network broadcast, and (b) one or more wall-display PCs that render the multi-tile view to the physical operations-room screen.

## 1.1  Deployment at a Glance

| Component | Purpose | Recommended Class |
| --- | --- | --- |
| Central server | Ingest, recording, clip render, database, web/API, network broadcast, future AI | Dual-socket 64-core, 256 GB RAM, GPU-accelerated |
| Video-wall PC | Render 32-channel multi-view to physical operations wall | Modern desktop with Intel UHD 770 iGPU or better |
| Storage subsystem | 30-day rolling recording archive plus hot data for immediate clip creation | 160 TB usable, tiered NVMe + HDD |
| Networking | Multicast satellite ingest plus network broadcast to viewer PCs | Managed L2 switch with IGMP snooping, 10/25 GbE server uplink |

# 2. Workload Overview

The infrastructure design responds to the following concurrent workloads that must run continuously and without contention:

32-channel UDP ingest. Thirty-two H.264/H.265 satellite feeds arrive via UDP multicast from the ground-station receivers, each averaging approximately three megabits per second.

24 × 7 recording. Every active channel is written to disk continuously as sixty-second MP4 segments with SHA-256 evidentiary checksums, kept on a thirty-day rolling window.

Live clip creation. Operators cut, watermark, and export clips from any past moment within the retention window while recording is in progress — the clip render must not compete with the recorder for I/O bandwidth.

Video-wall display. A physical wall in the operations room shows all thirty-two channels simultaneously via a dedicated wall PC that consumes hardware-accelerated decoders on its own GPU. The server does not decode for the wall in the baseline design.

Network broadcast. For viewer PCs on the wider network that cannot reach the multicast domain, the server transcodes each channel to a lower-bitrate H.264 variant and re-broadcasts on unicast RTSP — one always-warm transcoder per channel.

Web application. The Next.js front-end and Fastify API power operator access to recordings, clips, users, audit logs, and administration.

Future AI / analytics. Anticipated features include OCR of on-screen tickers, speech-to-text transcription, face and logo detection, and content indexing for search. These require CPU or GPU compute reserved from day one.

# 3. Central Server Specification

## 3.1  Compute (CPU)

Recommended: Dual-socket Intel Xeon Gold 6448Y (Sapphire Rapids) — thirty-two cores per socket for a total of sixty-four physical cores / one-hundred-and-twenty-eight threads at 2.1 GHz base and 4.1 GHz turbo.

Cost-effective alternative: Dual AMD EPYC 9354P — same core count, roughly thirty per-cent lower price, higher single-thread clock speed.

Core-budget breakdown:

| Workload | Cores |
| --- | --- |
| 32 × ffmpeg recording (source-copy mode, 24 × 7) | ~ 2 |
| Concurrent clip renders (peak 5 simultaneous exports) | ~ 5 |
| Postgres, Redis, Next.js, Fastify API services | ~ 4 |
| Server-side broadcast to network (32 × H.264 transcode to 720p) | ~ 15 – 20 |
| AI and analytics (OCR, transcription, detection — future) | ~ 15 – 20 |
| 50 % capacity margin for growth up to 48 channels | + 32 |
| TOTAL BUDGET | ≈ 64 physical cores |

## 3.2  Memory (RAM)

Recommended: 256 GB DDR5-4800 ECC Registered, populated across all channels for maximum bandwidth.

| Component | Allocation |
| --- | --- |
| Postgres (database + shared cache) | 32 GB |
| Redis (in-memory pipeline state) | 8 GB |
| Node.js services (API + web + workers) | 16 GB |
| ffmpeg processes (32 recorders + clip renders) | 8 GB |
| OS filesystem cache (accelerates archive playback) | 64 GB |
| AI / ML model workspace (future) | 64 GB |
| 50 % headroom | 64 GB |
| TOTAL | 256 GB |

## 3.3  GPU (Video Broadcast and AI Acceleration)

Recommended: NVIDIA L4 — 24 GB, PCIe Gen 4, single-slot, low-profile, seventy-watt passive.

Eight concurrent NVENC and eight concurrent NVDEC engines handle 32-channel transcode for the network broadcast tier at under ten per-cent GPU utilisation.

7,680 CUDA cores accelerate future AI inference (OCR, transcription, detection) without CPU contention.

Passive cooling and low profile allow use in any 2U server chassis with airflow.

Budget alternative: NVIDIA T400 or Intel Arc A310 (~ ₹15,000 – ₹18,000) — sufficient for broadcast transcode alone, insufficient for AI workloads.

## 3.4  Storage Architecture

A three-tier design places each workload on the storage class that matches its access pattern — hot data on NVMe for latency, bulk archive on high-density HDD for cost per terabyte.

| Tier | Purpose | Configuration | Usable Capacity |
| --- | --- | --- | --- |
| Boot | OS, applications, logs | 2 × 1 TB NVMe M.2 · RAID 1 (mirror) | 1 TB |
| Hot data | Postgres, Redis persistence, last 48 h of recordings, clip working area | 4 × 4 TB NVMe U.2 · RAID 10 | 8 TB |
| Bulk archive | Rolling 30-day recording archive plus exported clips | 12 × 16 TB SATA enterprise HDD (7200 RPM) · RAID 6 | 160 TB |

Storage capacity justification:

Source rate ≈ 3 Mbps average per channel = 32.4 GB per day per channel.

32 channels × 30 days = 31 TB of raw archive footprint.

160 TB usable gives a headroom multiplier of roughly 5×, protecting against future channel additions and retention-window extensions.

Hardware RAID controller: Broadcom (LSI) MegaRAID 9560-16i — sixteen-port SAS/SATA plus NVMe support, eight-gigabyte battery-backed write cache, hardware parity offload.

## 3.5  Network

Two × 25 GbE SFP28 ports bonded LACP — primary satellite ingest and PC network traffic.

One × 1 GbE — dedicated out-of-band management (iDRAC / IPMI).

Downstream switch must support IGMP snooping to prevent thirty-two multicast groups from flooding the office network.

## 3.6  Chassis, Power and Reliability

2U rack chassis, twelve × 3.5-inch hot-swap drive bays (Dell PowerEdge R760, HPE ProLiant DL380 Gen 11 or Supermicro AS-2015CS class).

Two redundant, hot-swap 1200 W 80-PLUS Platinum power supplies.

Front-panel LCD, out-of-band management, hardware Trusted Platform Module (TPM 2.0).

Rail kit for tool-less installation into a standard 42U rack.

# 4. Video-Wall PC Specification

The wall-display PC consumes the thirty-two channels directly via multicast and renders them onto the physical operations wall using its integrated or discrete GPU for hardware H.264 / H.265 decoding. A modest modern desktop is sufficient — the operator machine used in initial testing (Intel Core i5-14500 with UHD 770 integrated graphics) handles thirty-two concurrent 1080p decodes at well under twenty per-cent CPU utilisation.

## 4.1  Recommended Tiers

Tier 1 — Baseline Production (current test PC)

Intel Core i5-13500 or newer with integrated UHD 770 graphics.

16 GB DDR4 or DDR5 memory.

512 GB NVMe solid-state boot drive.

1 GbE network interface.

Cost band: approximately ₹80,000 – ₹1,00,000.

Tier 2 — Recommended Production (future-proof)

Intel Core i7-14700 (14-core / 20-thread) with integrated UHD 770.

32 GB DDR5-5600 memory.

1 TB PCIe Gen 4 NVMe boot drive.

2.5 GbE network interface.

Native support for four 4K monitors from iGPU.

Windows 11 Professional.

Cost band: approximately ₹1,80,000 – ₹2,20,000.

Tier 3 — High-End / Multi-Wall (scaling scenarios)

Intel Core i9-14900K (24-core / 32-thread).

Dedicated NVIDIA T400 or A400 (dedicated NVDEC — sixty-plus concurrent 1080p decodes).

64 GB DDR5 memory.

2 TB PCIe Gen 4 NVMe boot drive.

10 GbE network interface.

Support for multiple video-wall processors and 8K output.

Cost band: approximately ₹3,50,000 – ₹4,50,000.

# 5. Network Infrastructure

## 5.1  Recording LAN

The recording LAN carries satellite multicast traffic between the ground-station receivers, the central server and the wall PC.

Managed Layer-2 switch with IGMP snooping (Cisco Catalyst 9200/9300 · Aruba 2930 · MikroTik CRS series).

10 GbE uplink from switch to the central server.

1 GbE ports for each satellite receiver.

1 – 2.5 GbE port for the video-wall PC (total multicast traffic ≤ 100 Mbps for 32 concurrent 1080p streams).

Dedicated VLAN for satellite multicast traffic to prevent flooding of office LAN.

## 5.2  Broadcast VLAN (optional, for PCs off the recording LAN)

Server transcodes each channel to a lower-bitrate H.264 variant via GPU (NVIDIA L4).

Broadcast served via RTSP or HLS unicast to any authenticated PC on the corporate network.

Bandwidth: approximately 500 Kbps per channel × 32 channels = 16 Mbps per remote viewer.

# 6. Indicative Cost Summary

Costs are indicative reference figures based on typical Indian market pricing at document date. Client-side sourcing, vendor negotiation, and existing framework agreements will influence the final procurement figures. Software licences (Windows Server, database, etc.) are not included — the platform runs equivalently on Linux (Ubuntu Server LTS or AlmaLinux) with no functional loss.

| Line Item | Approx. INR |
| --- | --- |
| Server chassis (Dell PowerEdge R760 or equivalent) | 5,00,000 |
| CPUs — 2 × Intel Xeon Gold 6448Y (or AMD EPYC 9354P × 2) | 6,00,000 |
| 256 GB DDR5-4800 ECC Registered memory | 2,00,000 |
| NVIDIA L4 GPU (24 GB PCIe Gen 4) | 2,00,000 |
| Boot storage — 2 × 1 TB NVMe M.2 | 30,000 |
| Hot storage — 4 × 4 TB NVMe U.2 (RAID 10) | 2,00,000 |
| Bulk archive — 12 × 16 TB enterprise SATA HDD (RAID 6) | 3,50,000 |
| Broadcom MegaRAID 9560-16i controller + BBU | 50,000 |
| Server subtotal | ≈ 21,30,000 |
| Wall PC — Tier 2 recommended | ≈ 2,00,000 |
| Managed switch + rack cabling | ≈ 1,50,000 |
| 5 kVA online UPS with 30-minute run-time | ≈ 1,00,000 |
| GRAND TOTAL (indicative) | ≈ 26,00,000 |

# 7. Design Justification

This section explains the reasoning behind each major choice and why cheaper alternatives were not recommended — anticipating the questions the client will ask during review.

Sixty-four physical cores. A dual-socket 64-core system handles the 32-channel recorder and clip-render workload today, absorbs the network broadcast transcoding tier, and reserves half its capacity for the AI/analytics features scheduled for phase two. A smaller CPU forecloses those features and forces a hardware refresh before their release.

256 GB of RAM. The dominant memory consumer is the OS filesystem cache — its size determines how fast operators can browse and play back historical recordings. Under-provisioning at this line item degrades the daily user experience in the most visible workflow. AI models in the phase-two roadmap require an additional 40 – 60 GB of RAM at inference time.

NVIDIA L4 GPU. The GPU is what makes network broadcast to remote PCs feasible without doubling the CPU count. It also unlocks the AI roadmap without further capital investment. Without a GPU, remote consumption of the wall is either impossible or restricted to a handful of concurrent viewers.

160 TB of usable storage. Broadcast recordings are irreplaceable — a segment lost is a segment lost. Provisioning for 30 days at present channel count with a 5× multiplier absorbs future channel additions and any decision to extend retention beyond thirty days. RAID 6 with dual-drive fault tolerance protects against the specific failure mode most common in bulk HDD arrays: a rebuild-time second-drive failure.

Redundant power and 25 GbE networking. 24 × 7 continuous ingest cannot tolerate single-power-supply failure. 25 GbE at the server uplink provides headroom for 4K stream support the industry is moving to, without a switch replacement in the medium term.

# 8. Future Considerations

Doubling channel count to 64 requires only a proportional storage expansion — CPU and RAM headroom already accommodates it.

Adding a second physical wall in another operations room requires an additional Tier-2 or Tier-3 wall PC only; the central server is unaffected.

AI features (OCR ticker capture, speech-to-text transcription, face and logo recognition) are enabled by the GPU line item in the recommended specification. No additional hardware is required for phase-two feature rollout.

Hot / cold storage tiering can be introduced later with additional HDD shelves, extending retention beyond thirty days on the same chassis.

Off-site backup of the daily recording manifest (SHA-256 checksums plus segment metadata) is a low-cost addition (few tens of megabytes per day) that materially strengthens the evidentiary chain.

# 9. Contact and Follow-Up

Analysys will support the tender and procurement process, review vendor quotations against this specification, and confirm hardware compatibility before purchase. Deployment and commissioning services are covered under a separate statement of work.

For clarifications, please contact your Analysys account manager.

———————————————————————

End of Document · CMC Production Infrastructure Specification v1.0