BesTel Communications (Pvt) Ltd.

Pakistan | USA | KSA | UK | Spain  •  www.bestel.tech

TECHNICAL WRITE-UP & HIGH-LEVEL SYSTEM ARCHITECTURE

PSDP Project — Central Monitoring Unit (CMU) for IPTV Streams of 124 Channels

Acquisition • Encoding / Storage • API Exposure • Security • Scalability • Redundancy • 24×7 Operations

| Tender Reference | P28901 |
| --- | --- |
| Clarification Reference | F. No. 10-02/2022-E/PT, dated 2nd June 2026 — Item (v) |
| Procuring Agency | Directorate of Electronic Media & Publications (DEMP), Ministry of Information & Broadcasting, Government of Pakistan |
| Submitted by | BesTel Communications (Pvt) Ltd., Walton Road, Lahore, Punjab |
| Platform | Analysys — BesTel in-house IPTV Intelligence Platform (analysys.tech) |
| Document Date | 3rd June 2026 |

# 1.  Purpose & Scope of this Document

This technical write-up is submitted in response to clarification item (v) of the Procuring Agency’s letter F. No. 10-02/2022-E/PT dated 2nd June 2026, which requests “a technical write-up / high-level system architecture and methodology for the CMU IPTV streaming and API exposure, covering acquisition, encoding/storage, security, scalability, redundancy and 24/7 operations.”

The architecture presented here is not theoretical. It is a direct evolution of Analysys — BesTel’s production IPTV intelligence platform — which is already operating at parallel scale (125+ channels) for a Government of Pakistan client, and was first delivered to DEMP itself in 2014–15. The parameters, protocols and operating procedures stated below reflect the in-production stack (FFmpeg + MediaMTX + hls.js + MongoDB), which materially reduces delivery risk and shortens time-to-go-live.

The document is organised against the exact sub-criteria DEMP has asked us to address, so each requirement can be located and scored without ambiguity:

- Solution methodology and design principles (Section 2)

- IPTV acquisition modes (Section 3)

- Encoding / transcoding and storage (Section 4)

- High-level CMU architecture and end-to-end data flow (Section 5)

- Secure REST API exposure and the video wall (Section 6)

- Security — defence-in-depth (Section 7)

- Scalability (Section 8)

- Redundancy and disaster recovery (Section 9)

- 24×7 operations, SLA and incident management (Section 10)

- Standards compliance and implementation methodology (Sections 11–12)

# 2.  Solution Methodology & Design Principles

The CMU is designed as a centralized facility that continuously ingests licensed feeds of 124 national and international channels, transcodes them to standards-based renditions, and exposes both the live streams and their metadata as secure, documented REST APIs for onward consumption by the Ministry’s AI-enabled monitoring and analytics applications. The platform runs 24×7 with redundancy, disaster recovery and measurable service levels.

The design is governed by six non-negotiable principles, each of which is reflected in DEMP’s RFP:

- Carrier-grade resilience — a minimum of three (3) independent acquisition sources per channel with automatic, sub-2-second failover and full audit logging.

- Standards-first delivery — MPEG-TS over IP, RTP/RTSP, RTMP and HLS for ingest and onward delivery; ABR transcoding for multi-bitrate distribution; zero proprietary lock-in.

- Secure, API-first exposure — OAuth 2.0 / JWT-secured REST endpoints with fine-grained authorization, rate-limiting and complete audit trails; every platform function is reachable through the API, not locked inside the UI.

- Browser-native operations — a multi-layout video wall with six or more configurable layouts, accessible from any modern browser without proprietary client software.

- Deployment flexibility — the identical platform can be delivered on-premise at the CMU data centre, as a fully-managed SaaS (zero hardware from DEMP), or hybrid, without architectural change.

- Horizontal scalability — every tier scales out independently; capacity headroom for channels beyond 124 is built into the initial sizing.

Delivery follows an Agile / milestone-gated methodology: a fifteen (15) calendar-day core deployment cycle to operational go-live, followed by a fifteen (15) day stabilization and acceptance period, then continuous operations and 24×7 support for the full contract tenure.

# 3.  IPTV Acquisition Modes

Acquisition is hybrid by default — the platform never depends on any single bearer technology. Each of the 124 channels is provisioned with at least three independent sources drawn from the following modes:

| Acquisition Mode | Method | Output / Notes |
| --- | --- | --- |
| IP / Multicast | MPEG-TS over UDP/RTP multicast from telco aggregation networks, via dual-NIC ingest servers | IGMPv3 join, redundant paths |
| RTMP / RTSP Push | For rights-holder push uplinks rather than pulls | Encoder/IP-camera uplinks |
| OTT / HLS Pull | Licensed OTT or partner HLS origins | Secondary/tertiary source |

BesTel has collaborated with KK Networks (Pvt) Ltd. — a nationwide FTTH/IPTV provider with infrastructure across all 14 telecom regions of Pakistan — to provide carrier-grade source diversity, last-mile redundancy and on-ground support. Additional licensed streams are sourced from other IPTV providers (and, where licensed, PTCL / Nayatel), so that the three-source rule per channel is satisfied from genuinely independent paths.

# 4.  Encoding / Transcoding & Storage

## 4.1  Encoding / Transcoding

| Parameter | Specification |
| --- | --- |
| Video codec | H.264 (libx264), ultrafast preset + low-latency tuning; H.265/HEVC where downstream consumers support it |
| Audio codec | AAC-LC, dual channel, 128 kbps default |
| ABR ladder | 1080p @ 4 Mbps / 720p @ 2.5 Mbps / 480p @ 1 Mbps — configurable per channel |
| Segment length | 4-second HLS segments (configurable down to 2 s for low-latency profiles) |
| Hardware acceleration | Optional NVIDIA NVENC / Intel Quick Sync for higher channel density per encoder node |
| Output formats | HLS, RTMP, MPEG-TS, and optionally Low-Latency HLS (LL-HLS) / LL-DASH |

## 4.2  Storage

| Parameter | Specification |
| --- | --- |
| Rolling buffer | 30-day default per channel — configurable per channel and per category |
| Container format | Fragmented MP4 (fMP4) aligned with HLS segments for seamless seek and clip extraction |
| Capacity sizing | ≈120 TB raw for 124 channels @ avg 3 Mbps, 30-day retention; 200–250 TB provisioned for headroom + replication |
| Storage backend | Clustered QNAP NAS (or equivalent), RAID-6 + hot-spare; redundant 10GbE network paths |
| Metadata store | MongoDB replica set for channel/source metadata, events, RBAC and audit logs |
| Archive openness | fMP4 / CMAF is readable by any standards-compliant tooling — DEMP is never dependent on BesTel for archive access |

## 4.3  Performance Targets

| Metric | Target |
| --- | --- |
| Platform availability | ≥ 99.9% |
| Per-channel availability (excluding source-side outages) | ≥ 99.5% |
| Failover switch-over time | ≤ 2 seconds |
| Glass-to-glass latency (standard HLS) | ≤ 6 seconds |
| Glass-to-glass latency (LL-HLS) | ≤ 2 seconds |
| API p95 latency (metadata endpoints) | ≤ 300 ms |
| API error rate (nominal load) | ≤ 0.1% |
| Video wall cold-load | ≤ 5 seconds |
| Recording continuity | ≥ 99.9% segments persisted |

# 5.  High-Level CMU Architecture & Data Flow

The CMU platform is structured as seven horizontally-scalable tiers, each independently deployable, replaceable and capable of being scaled out without architectural re-design.

| # | Tier | Purpose |
| --- | --- | --- |
| 1 | Acquisition Tier | IP/multicast endpoints with redundant network paths |
| 2 | Failover & Probing Tier | Continuous health probing of every source per channel; automatic switch-over to the next healthy source |
| 3 | Encoding / Transcoding Tier | FFmpeg-based real-time transcoding to ABR rendition ladders |
| 4 | Streaming Origin Tier | MediaMTX cluster publishing HLS / RTMP / LL-HLS |
| 5 | API & Application Tier | Node.js + Express microservices delivering REST APIs, webhooks, video-wall UI and admin console |
| 6 | Storage Tier | Rolling fMP4 recording archive (NAS) + MongoDB metadata store |
| 7 | Observability & Security Tier | Centralised ELK logging, Prometheus/Grafana metrics, WAF, IDS/IPS and audit trail |

## 5.1  End-to-End Data Flow (per channel, with 3-source failover)

- Acquisition — Source A (satellite/IRD), Source B (IP/multicast) and Source C (IP/multicast) deliver the live signal.

- Health probing — each source is evaluated every 1–2 s against signal-presence, bitrate-floor, PCR continuity and (optionally) blank-frame heuristics.

- Failover — if the active source fails the probe, the orchestrator promotes the next healthy source within ≤ 2 s; the downstream URL stays unchanged so consumers see no interruption.

- Transcoding — FFmpeg produces three ABR renditions per channel; segment length 4 s (default).

- Publishing — transcoded streams are pushed via RTMP to the MediaMTX origin (:1935) and republished as HLS (:8888).

- Distribution — authorized clients retrieve streams over HTTPS via the API gateway with authentication, authorization and rate-limiting.

- Recording & logging — a recorder writes rolling fMP4 segments to the NAS; every lifecycle event is logged centrally and is independently auditable.

# 6.  Secure REST API Exposure & Video Wall

## 6.1  Core API Endpoints (REST + Webhooks)

| Endpoint | Function |
| --- | --- |
| GET /api/v1/channels | List of all channels with metadata (id, name, language, category, region, current source, status) |
| GET /api/v1/channels/{id}/playlist.m3u8 | Signed, authenticated HLS playlist URL |
| GET /api/v1/channels/{id}/status | Real-time source, last failover, uptime and current bitrate |
| GET /api/v1/recordings | Recorded segment lookup by channel and time window |
| GET /api/v1/recordings/{id}/clip | On-demand clip generation |
| GET /api/v1/events | Auditable platform events (failovers, access, errors) |
| GET /api/v1/metadata/{channel}/transcript | Speech-to-text transcript stream (Urdu / English) |
| Webhook subscriptions | Failover events, content alerts, recording completions — HMAC-SHA256 signed payloads |

Developer artefacts delivered at go-live: an OpenAPI 3.1 specification, a Postman collection, and Node.js + Python SDKs with sample integration code — reducing downstream integration effort to hours. The API is versioned (/api/v1/) with a published deprecation policy so new versions co-exist with legacy versions for staged downstream migration.

## 6.2  Video Wall

A browser-native React + Vite + hls.js single-page application with six or more configurable layouts — 1×1, 2×2, 3×3, 4×4, 5×5, a 6×4/8×4 large wall, and a PiP-focus mode. Layouts are saved per user and recalled on login. Closed captions, on-screen channel labels and a synchronized time-strip are configurable globally. No proprietary client software is required.

# 7.  Security — Defence-in-Depth

| Layer | Control |
| --- | --- |
| Perimeter | Next-Gen Firewall + IDS/IPS + WAF + geo-IP filtering + DDoS scrubbing |
| Transport | TLS 1.2+ on every HTTP/HTTPS path; HSTS; modern cipher suites only |
| At-rest encryption | AES-256 on recording storage and database backups |
| Identity | OAuth 2.0 + JWT; optional OIDC federation with the DEMP/Ministry IdP; MFA mandatory for administrative roles |
| Authorization (RBAC) | Predefined roles (Viewer / Analyst / Operator / Admin / Auditor) plus custom roles |
| Rate limiting | Per-tenant token-bucket limits per endpoint group; HTTP 429 with Retry-After |
| URL signing | Time-limited signed HLS playlist URLs prevent unauthorized re-sharing |
| Host hardening | CIS-aligned OS hardening, kernel auditing, fail2ban on management hosts |
| Vulnerability mgmt | Monthly scans, quarterly patch cycle, emergency CVE patching |
| Audit trail | Immutable write-once logging of all administrative actions; exportable to the DEMP audit authority |
| Network segmentation | DMZ / Acquisition / Processing / Storage / Management / DR zones with east-west firewall rules |

# 8.  Scalability

- Horizontal scale-out on every tier — ingest, encoder, origin or API nodes are added without architectural redesign.

- Stateless application tier — services can be killed and re-instantiated without data loss; auto-scaling under load.

- Optional caching tier — CDN or internal cache layer for high fan-out distribution.

- Capacity headroom built into the initial sizing for additional channels beyond 124.

- Docker + Compose / Kubernetes orchestration for consistent, repeatable scaling operations across bare-metal, virtualized or cloud infrastructure.

# 9.  Redundancy & Disaster Recovery

- 3-source failover per channel with sub-2-second switch-over.

- Active-active streaming origin — each channel is published to ≥ 2 MediaMTX nodes.

- Active-active ingest — dual ingest nodes with health-based traffic steering.

- MongoDB replica set (Primary + Secondary + Arbiter) for metadata high-availability.

- NAS RAID-6 + hot-spare for recording survivability.

- Asynchronous DR replication to a secondary site — RTO 4 hours; RPO 15 minutes (metadata) / 1 hour (recordings).

- UPS + diesel-generator backup at the CMU facility (on-premise mode).

- Backup policy — daily incremental, weekly full, quarterly tested restore drills.

# 10.  24×7 Operations, SLA & Incident Management

## 10.1  Operations

- Pakistan-based NOC manned 24×7 by Tier-1 / Tier-2 engineers — no time-zone or escalation friction.

- Dedicated DEMP dashboard — per-channel health, per-node metrics, API throughput, storage utilization and incident queue.

- Preventive maintenance — monthly OS patching, weekly health audits, quarterly DR drills.

- Change management — maintenance windows scheduled outside prime-time broadcast hours with advance notice to DEMP.

- Monthly Service Review with a formal SLA report.

## 10.2  Incident Management SLA

| Severity | Definition | Response | Resolution |
| --- | --- | --- | --- |
| S1 — Critical | Platform-wide outage / > 25 channels down | 15 minutes | 4 hours |
| S2 — High | 1–25 channels down or API degraded | 30 minutes | 8 hours |
| S3 — Medium | Single source failure with failover active | 2 hours | 1 business day |
| S4 — Low | Cosmetic or feature request | 1 business day | Next maintenance window |

# 11.  Standards Compliance

The proposed solution is built exclusively on open IETF, ISO/IEC, ITU-T and W3C standards — every protocol is publicly documented, interoperability-tested and supported by multiple independent implementations. There is no proprietary lock-in.

| Domain | Standards Applied |
| --- | --- |
| Broadcast / streaming | MPEG-TS over UDP/RTP (ISO/IEC 13818-1), HLS (RFC 8216), MPEG-DASH (ISO/IEC 23009-1), LL-HLS, RTMP/RTMPS, RTSP (RFC 7826), H.264 (ITU-T H.264), H.265 (ITU-T H.265), AAC-LC (ISO/IEC 14496-3), fMP4 / CMAF |
| API | REST, OpenAPI 3.1, JSON (RFC 8259), JSON Schema 2020-12, HTTP/1.1 + HTTP/2 (RFC 7230 / 9113), HMAC-SHA256 webhook signing (RFC 2104) |
| Security | TLS 1.2/1.3 (RFC 5246/8446), OAuth 2.0 (RFC 6749), JWT (RFC 7519), OIDC Core 1.0, mTLS (RFC 8705), CIS Benchmarks, OWASP Top 10 / API Top 10, AES-256 (FIPS 197), SHA-256 (FIPS 180-4) |
| Operations | Syslog (RFC 5424), Prometheus exposition, OpenTelemetry 1.x, NTP/PTP (RFC 5905 / IEEE 1588) |

# 12.  Implementation Methodology & Timeline

| Phase | Key Activities | Duration |
| --- | --- | --- |
| Mobilization & Design | Kick-off, detailed design freeze, source/licensing confirmation, environment provisioning | Day 0–3 |
| Acquisition & Encoding | IRD/multicast onboarding, 3-source mapping, ABR encoder configuration | Day 3–8 |
| Streaming, API & Video Wall | MediaMTX origin, REST API + webhooks, video-wall layouts, RBAC | Day 8–12 |
| Hardening & Go-Live | Security hardening, recording/DR verification, FAT, operational go-live | Day 12–15 |
| Stabilization & Acceptance | Monitoring, tuning, SAT, documentation & training handover | Day 15–30 |
| Operations & Support | 24×7 NOC, preventive maintenance, monthly SLA reviews | Full tenure |

The 15-day-to-go-live plus 7–15-day stabilization timeline is realistic, measurable, and underwritten by previous BesTel deliveries of the Analysys platform to Government of Pakistan clients at comparable scale.

# 13.  Conclusion

This write-up demonstrates a complete, standards-based and production-proven architecture for the CMU — covering acquisition, encoding/storage, API exposure, security, scalability, redundancy and 24×7 operations as requested. Because the design is a battle-tested evolution of the Analysys platform already operating with a Government of Pakistan client, DEMP benefits from reduced delivery risk, a short time-to-go-live, full in-house engineering accountability, and a clear AI/analytics roadmap (ContentSense, InstaClip, TVC Tracking, speech-to-text), with no proprietary vendor lock-in.

BesTel remains available to demonstrate the platform, supply any additional information, or appear before the evaluation committee at the Procuring Agency’s convenience.

For and on behalf of BesTel Communications (Pvt) Ltd.

Mohammad Mujtaba

Chief Revenue Officer

mohammad.mujtaba@bestel.tech

BesTel Communications (Pvt) Ltd.  |  https://bestel.tech